# Race Planner Parameter Study

Date: 2026-04-14 23:31:02

Study design: 10 clean-restart simulator runs on the Spielberg map. Each run used one parameter group and recorded the first completed lap time.

| Group | Configuration | Changed Parameters | Lap 1 Time (s) | Status | Notes |
| --- | --- | --- | --- | --- | --- |
| g01_baseline_fast | Baseline Fast | baseline | 66.38 | Completed | Reference configuration using the repo Spielberg fast profile. |
| g02_conservative_speed | Conservative Speed | max_speed=7.0, straight_speed=6.6, fast_curve_speed=5.6, medium_speed=4.3, corner_speed=2.5 | 69.47 | Completed | Lower top and corner speeds to favor stability over lap time. |
| g03_aggressive_speed | Aggressive Speed | max_speed=8.5, straight_speed=8.0, fast_curve_speed=6.8, medium_speed=5.2, corner_speed=3.0, lat_accel_limit=10.2, accel_limit=6.4, decel_limit=10.3, brake_decel=8.9 | 61.21 | Completed | Raise speed limits and acceleration to test a faster but riskier setup. |
| g04_high_clearance | High Clearance | preferred_wall_clearance=0.9, safety_margin=0.22, clearance_weight=3.1, center_weight=1.7 | 68.34 | Completed | Increase wall and safety margins to keep the car farther from obstacles. |
| g05_low_clearance | Low Clearance | preferred_wall_clearance=0.64, safety_margin=0.14, clearance_weight=2.4, progress_weight=2.5 | 66.57 | Completed | Reduce clearance margins to allow tighter cornering lines. |
| g06_long_horizon | Long Horizon | min_horizon=2.8, max_horizon=6.6, lookahead_speed_gain=0.36 | 67.87 | Completed | Plan farther ahead with more speed-sensitive lookahead. |
| g07_short_horizon | Short Horizon | min_horizon=2.0, max_horizon=4.9, lookahead_speed_gain=0.22 | 65.33 | Completed | Use a shorter planning horizon for tighter local reactions. |
| g08_smoother_steering | Smoother Steering | steering_smoothing=0.58, corner_steering_smoothing=0.38 | 65.47 | Completed | Increase steering smoothing and reduce sudden steering steps. |
| g09_sharper_steering | Sharper Steering | steering_smoothing=0.38, corner_steering_smoothing=0.2 | 66.49 | Completed | Reduce smoothing to let the planner respond more aggressively. |
| g10_ml_disabled | ML Disabled | enable_ml_tuner=False | 68.01 | Completed | Disable the ML tuner to compare learned adaptation versus fixed rules. |

Best observed lap: Aggressive Speed (g03_aggressive_speed) with 61.21s.